ALL INDIA INSTITUTE OF SPEECH AND HEARING, MYSORE – 6

ESTIMATE

Name of the work: Construction of underground sump at patients accommodation block                                (old gents hostel building), AIISH Naimisham campus, Mysore.

| Sl No. | Description of work | Qty. | Rate in Rupees | Amount in Rupees |
| --- | --- | --- | --- | --- |
| 1. | Earthwork excavation for foundation of buildings, culverts, water supply, sanitary lines and electrical conduits either in pits or in trenches 1.5 m and above in width, in ordinary soil not exceeding 1.5m in depth including dressing the bottom and sides of pits and trenches, stacking the excavated soil clear from edges of excavation with lead upto 50m after breaking of clods complete as per specification No.KBS 2.1(a)/2.3.5 1 x 3.60 x 2.60 x 0.30 = 2.80 Cum SR 2014-15 Page No 5, S l no 2.3 163.00 + 3% = 167.89 b) in Hard soil spec No.KBS 2.1(b)/2.3.5 1 x 3.60 x 2.60 x 0.60 = 5.61 Cum SR 2014-15, Page no 5, Sl no 2.4 - 3% = 252.35 - In Soft rock without blasting- spec No.KBS 2.1(c)/2.9.4/2.18/2.19 1 x 3.60 x 2.60 x 0.60 = 5.61 Cum SR 2014-15, Page no 5, Sl no 2.6 764.00 + 3% = 786.92 | 2.80 Cum 5.61 Cum 5.61 Cum | 167.89 252.35 786.92 | 470.09 1,415.68 4,414.62 |
| 2. | Filling available excavated earth (excluding rock) in sides of foundation up to plinth in layers not exceeding 20 cms in depth, compacting each deposited layer by ramming after watering with lead up to 50 m & lift up to 1.5 m, including cost of all labour complete as per specification No KBS 2.9. 1 x 10.00 x 0.60 x 1.50 = 9.00 Cum 110 + 3% = 113.30 SR 2014-15 Page No 6, S l no 2.10 | 9.00 Cum | 113.30 | 1,019.70 |
| 3. | Providing and laying in position plain cement concrete of mix 1:4:8 with OPC cement @ 180 kgs, with 40mm and down size graded granite metal coarse aggregates @ 0.85 cum and fine aggregates @ 0.57 cum machine mixed, concrete laid in layers not exceeding 15cms. thick, well compacted, in foundation including cost of all materials, labour, HOM of machinery, curing complete as per specification No KBS 4.1,4.2 3.30 x 2.30 x 0.15 = 1.13 Cum 4,763.00 + 3 % = 4,905.89 SR 2014-15 Page no12, Sl no 4.3 | 1.13 Cum | 4,905.89 | 5,543.65 |

| Sl No. | Description of work | Qty. | Rate in Rupees | Amount in Rupees |
| --- | --- | --- | --- | --- |
| 4. | Providing and laying in position reinforced cement concrete of design mix M 20 with OPC cement @ 320 kgs, with 20mm and down size graded granite metal coarse aggregates @ 0.69 cum and fine aggregates @ 0.46 cum, with super plasticizer @ 3 lts confirming to IS 9103-1999 Reaffirmed-2008, machine mixed, concrete laid in layers not exceeding 15cms thick, vibrated for all works in ground floor level for roof slabs, staircase, lintels, and beams retaining walls, return walls, walls (any thickness) including attached plasters, columns, pillars, posts, struts, buttresses, string or lacing courses, parapets, coping, bed blocks, anchor blocks, plain window cills, fillets etc., including cost of all materials, labour, curing HOM of machinery, curing, complete as per specification (Excluding cost of steel, fabrication & plastering charges) specification No KBS 4.1,4.6. 1 x 10.00 x 1.50 x 0.20 = 3.00 Cum 2 x 3.00 x 2.00 x 0.20 = 2.40 Cum Hau 4x½ x 10.00x 0.30 x 0.30 = 1.80 Cum Ded 1 x 0.60 x 0.60 x 0.20 = (-) 0.07 Cum SR 2014-15 Page No 14, S l no 4.13 6,030.00 + 3% = 6,210.90 | 7.13 Cum | 6,210.90 | 44,283.71 |
| 5. | Providing and removing centering, shuttering, strutting, propping etc., and removal of form work for vertical surface such as walls at any thickness, including attached pilasters, buttresses, plinth and string courses cost of all materials, labour complete as per specifications No.KBS 4.6.2 (1 x 2) x 10.00 x 1.50 = 30.00 Sq m SR 2014-15 Page No 16, S l no 4.31 271.00 + 3% = 279.13 | 30.00 Sqm | 279.13 | 8,373.90 |
| 6. | Providing and removing centering, shuttering, strutting, propping etc., and removal of form work for flat surface such as suspended floors, roofs (applicable for this work estimate), landings, balconies and likes, thickness above 200 mm, including cost of all materials, labour complete as per specifications specification No.KBS 4.6.2 2 x 3.00 x 2.00 = 12.00 Sqm SR 2014-15 Page No 16, S l no 4.30 356.00 + 3% = 366.68 | 12.00 Sqm | 366.68 | 4,400.16 |

| Sl No. | Description of work | Qty. | Rate in Rupees | Amount in Rupees |
| --- | --- | --- | --- | --- |
| 7. | Providing T.M.T. Steel reinforcement for R.C.C work including straightening, cutting, bending, hooking, placing in position, lapping and (or welding wherever required) tying with binding wire and anchoring to the adjoining members wherever necessary complete as per design tying with binding wire as per design (laps, hooks and wastage shall not be measured and paid) cost of materials, labour, HOM of machinery complete as per specification No 4.6.3 for TMT bars Fe 500 SR 2014-15 Page No 19, S l no 4.46.2 73,851.00 + 3% = 76,066.53 | 0.67 MT | 76,066.53 | 50,964.58 |
| 8. | Providing 20mm thick cement plaster in single coat with cement mortar 1:3, to stone masonry & concrete surface ( for raft slab & walls in this work estimate) including rounding off corners wherever required, smooth rendering, providing and removing scaffolding, including cost of materials, labour, curing complete as per specification. 1 x 10.00 x 1.50 = 15.00 2 x 3.00 x 2.00 = 12.00 Sqm 247 + 3% = 254.41 SR 2014-15 Page No116, S l no 15.18 | 27.00 Sqm | 254.41 | 6,869.07 |
| 9. | Extra for providing and mixing waterproofing compound in cement plaster for brick masonry work (for raft slab & walls in this work estimate) at one kg per bag or in the proportion recommended by the manufacturers, for cement mortar 1:3, 20 mm thick cost of materials complete as per specifications. Qty same as Sl.No 8 12.50 + 3% = 12.87 SR 2014-15 Page No116, S l no 15.28 | 27.00 Sqm | 12.87 | 347.49 |
| 10. | Providing and fixing top manhole cover of size 60 cms x 60 cms using angle iron of size 32 mm x 32 mm x 6 mm with M.S Sheet 14 Gauge supported with 12mm square rods all-round & diagonals including cost of primer coat & 2 coats of paint, materials, labour, HOM of machinery complete as per specifications. | 1 No | 4,000.00 MR | 4,000.00 |
| 11. | Providing & fixing air vents with 3” PVC pipe as per directions & as per site condition. | 4 No | 800.00 MR | 3,200.00 |

| Sl No. | Description of work | Qty. | Rate in Rupees | Amount in Rupees |
| --- | --- | --- | --- | --- |
| 12. | Providing and fixing chlorinated Poly Venyl Chloride (CPVC) pipes conforming to IS 15778 having thermal stability for hot and cold water supply including all CPVC plain and brass threaded fittings, this includes jointing of pipes and fittings with one step CPVC solvent cement, trenching, refilling & testing of joints complete (External work) – 25 mm nominal OD pipes for delivery line from sump to the water supply line connecting overhead tank 160.00 + 3% = 164.80 SR 2014-15 Page No 99, S l no 13.87.3 | 30.00 mt | 164.80 | 4,944.00 |
|  | Miscellaneous works such as providing and fixing PVC foot steps for maintenance purpose and round off |  |  | 933.40 |